STEPS TO GENERATE MLC CONFIGURATION FILE FOR IMU

Overview:

- IMU: ASM330LHHX

- Exposes ACC and GYRO sensors

Step 1: Sensor data collection

Please find below steps to collect gyro/Accel sensor data using the high_freq_sensor_tool binary:

- Push the Binary attached high_freq_sensor_tool binary to the device path:

/vendor/bin

- If an existing recorded file is present, remove it using the following commands in an ADB shell:

$ su

# rm -rf /data/sensor_gyro.txt

# rm -rf /data/vendor/sensor/*

- Run the Tool

Start the tool using the command below:

# high_freq_sensor_tool -c 1,1,0,0

Parameters:

- 1 → Sensor type: Accelerometer

- 1 → Enable sensor

- 0 → Default delay (nanoseconds)

- 0 → Default latency (nanoseconds)

- Perform Vehicle Test

Once the tool is running, start the vehicle and perform the following driving scenarios:

- Normal road driving

- Bumpy roads or speed breakers

- Sloped roads

This will help us to know how IMU data under various conditions.

- Collect Data After each test (adb pull):

- Retrieve /data/sensor_gyro.txt (Even though you have selected accelerometer, the log file will be named sensor_gyro.txt so, don’t get confused and clear the logs once pulled after collecting a particular sensor data)

- Collect contents from /data/vendor/sensor/

- Clear Logs Before Next Test

Before starting a new test, clear previous logs using:

$ su

# rm -rf /data/sensor_gyro.txt

# rm -rf /data/vendor/sensor/*

Sample data: The values are in m/s^2 unit.

data: [1,0,239835424936,-0.038000,0.238000,9.770000,3,1808679.000000]

data: [1,0,239837721875,-0.064000,0.239000,9.766000,3,1803436.125000]

data: [1,0,239840019796,-0.086000,0.228000,9.766000,3,1816217.125000]

data: [1,0,239842315721,-0.099000,0.215000,9.773000,3,1836709.250000]

Data overview:

ex: data: [1,0,239835424936,-0.038000,0.238000,9.770000,3,1808679.000000]

1 -> Sensor Type
0 -> reserved0

239835424936 -> Timestamp
-0.038000 -> ACC_X (m/s^2)
0.238000 -> ACC_Y (m/s^2)
9.770000 -> ACC_Z (m/s^2)
3 -> Sensor Status
1808679.000000 -> Avg Latency

Step 2: Format the above data in way compatible with MEMS Studio

Expected format sample: STMems_Machine_Learning_Core/application_examples/asm330lhhx/Tow detection/datalogs/Back_flatbed_Tow.txt at master · STMicroelectronics/STMems_Machine_Learning_Core

For our use-case, use the parser.py for format the collected data to expected format

Find the python script :  here

command to execute: python3 parser.py  /path/to/collected/data.txt  /path/to/expected/data.txt

Sample formatted data:

A_X [mg]	A_Y [mg]	A_Z [mg]

-958	-2724	3750

-481	-1353	2373

-240	-668	1683

-119	-324	1338

Step 3 Pre-requisites:

- We are going to train a model which uses Decision Tree Algorithm, a supervised learning algo which expects data and class labels.

- So, when feeding the formatted data in MEMS studio, be mindful in assigning the class label to respective data.

Note: Enter Adv Features in MEMS studio and then follow the below steps

Step 3: Setting Data Patterns in MEMS STUDIO

- Select the IMU device (i.e, ASM330LHHX)

- Set the workspace path

- Load the formatted datalog (i.e: .txt file which was generated from parser.py) and enter the class label name. Finally click on “Load”

Ex: acc_park.txt -> class label: PARKING
      acc_backwheel.txt -> class label: BACK_WHEEL

Step 4: Setting AFS tool

- Set the Output Data Rate accordingly. (Ex: 12.5Hz)

- Set what sensor type data was fed in Data Patterns section. (Ex: accelerometer_only)

- Set the accelerometer sensor configs (Ex: Full scale: 4g and ODR: 12.5Hz)

Step 5: Setting ARFF generation

- Set the path for features.arff file

- In Features section,

- Add the features which needs to be acted upon the data fed to the MEMS studio as shown in the below Image

- Click on Generate ARFF File button

Step 6: Generating Decision Tree

- Enter the number of decision trees to be generated (Based on the fine-tuned value)

- Click on Generate Decision Tree button to generate tree and heat map for all the decision trees.

Step 7: Generating mlc.json file in Config Generation section

- Make sure the class output values are unique for each class in MetaClassifier section

- If yes, then click on Generate Config File button

- This downloads the mlc.json in the path specified.